Project Robot / manipulator design with 5 degrees of freedom

Hop Nguyen

404839

Group:03

Table of Contents

1.State of art	3

2.Mechanism	5

3.FOWARD KINEMATICS	6

3.1 Mechanism	6

3.2 Kinematic table	6

3.3 Obtained Results	6

4.Inverse Kinematic	9

4.1 MAPLE CODE AND POSSIBLE SOLUTIONS FOR GIVEN POSITON	9

4.2 THE SOLUTION FOR θ6	10

5.Simple control	11

5.1 ROS and V-REP	11

5.2 Arduino with Servos	12

6.Model	13

7.APPENDIX	14

7.1 MATLAB CODE	14

7.2 ROSNODE CODE	15

7.3 ARDUINO CODE	18

# 1.State of art

In this project I will construct robot arm (T050000)

One of the most common and typical 5DOF robotic arms in the factory today. This mechanism is flexible and can be combined with automation to quickly and easily complete work that requires 2-3 people to complete.

With the machines' ability to lift heavy objects, they are used in auto plants, manufacturing materials or freight.

The construction of the robot can be adapted to be placed on a driving system based on linear guides and a high precision toothed bar drive. Thanks to this, the robot gains an additional axis of movement along which it can move with the load at almost any distance

I have tried to find some similar mechanism in the factories and laboratories:

# 2.Mechanism

Drawings of construction:

Degrees of freedom:

5 elements.

5 pair of class 1.

W= 6*5 - 5*5 = 5 degrees.

-Manipulators having three rotary joints are included in the group of anthropomorphic manipulators.

# 3.FOWARD KINEMATICS

3.1 Mechanism

X5

Y4

Y3

Y2

Y1

Y0

d6

a4

a3

a2

a1

Z6

Z5

Y5

Z4

Z3

Z2

Z1

Z0

X6

X4

X3

X2

X1

X0

Y6

## 3.2 Kinematic table

| System | θi | di | ai | αi |
| --- | --- | --- | --- | --- |
| 1 | 0 | 0 | a1 | α1,var |
| 2 | θ2,var | 0 | a2 | 0 |
| 3 | θ3,var | 0 | a3 | 0 |
| 4 | θ4,var | 0 | a4 | 0 |
| 5 | 90 | 0 | 0 | 90 |
| 6 | θ6,var | d6 | 0 | 0 |

## 3.3 Obtained Results

A1 =

[1,           0,            0, a1]

[0, cos(alpha1), -sin(alpha1),  0]

[0, sin(alpha1),  cos(alpha1),  0]

[0,           0,            0,  1]

A2 =

[cos(theta2), -sin(theta2), 0, a2*cos(theta2)]

[sin(theta2),  cos(theta2), 0, a2*sin(theta2)]

[          0,            0, 1,              0]

[          0,            0, 0,              1]

A3 =

[cos(theta3), -sin(theta3), 0, a3*cos(theta3)]

[sin(theta3),  cos(theta3), 0, a3*sin(theta3)]

[          0,            0, 1,              0]

[          0,            0, 0,              1]

A4 =

[cos(theta4), -sin(theta4), 0, a3*cos(theta4)]

[sin(theta4),  cos(theta4), 0, a3*sin(theta4)]

[          0,            0, 1,              0]

[          0,            0, 0,              1]

A5 =

[0     0     1     0]

[1     0     0     0]

[0     1     0     0]

[0     0     0     1]

A6 =

[cos(theta6), -sin(theta6), 0,  0]

[sin(theta6),  cos(theta6), 0,  0]

[          0,            0, 1, d6]

[          0,            0, 0,  1]

T06 =

[-sin(theta2+theta3+theta4)*cos(theta6),                                         sin(theta2+theta3+theta4)*sin(theta6),                              cos(theta2+theta3+theta4),                                                  a1+a3*cos(theta2+theta3)+a2*cos(theta2)+a3*cos(theta2+theta3+theta4)+ d6*cos(theta2+theta3+theta4)]

[cos(theta2+theta3+theta4)*cos(alpha1)*cos(theta6)-sin(alpha1)*sin(theta6),                                                     -sin(alpha1)*cos(theta6)-cos(theta2+theta3 +theta4)*cos(alpha1)*sin(theta6), sin(theta2+theta3+theta4)*cos(alpha1),                  cos(alpha1)*(a3*sin(theta2+theta3)+a2*sin(theta2)+a3*sin(theta2+theta3+theta )+d6*sin(theta2+theta3+theta4))]

[cos(alpha1)*sin(theta6)+cos(theta2+theta3+theta4)*sin(alpha1)*cos(theta6),   cos(alpha1)*cos(theta6)-cos(theta2+theta3+theta4)*sin(alpha1)*sin(theta6), sin(theta2+theta3+theta4)*sin(alpha1),                  sin(alpha1)*(a3*sin(theta2+theta3)+a2*sin(theta2)+a3*sin(theta2+theta3+theta4)+d6*sin(theta2+theta3+theta4))]

[0,                                                                                                                                                             0,                                                                                  0,                                                                                                                            1]

# 4.Inverse Kinematic

For calculation of forward kinematic problem, I used a MAPLE software.

For my calculation I took equations from the T05 matrix to determine 1 2, 3 and 4  because this matrix reduce calculations and the A6 matrix determines only the orientation of the end point (and moves the axis in z-axis but this change can be also given before rotation in A6 or just after the A5 matrix without any consequences).

## 4.1 MAPLE CODE AND POSSIBLE SOLUTIONS FOR GIVEN POSITON

Software solves equations and shows all solutions (a lot of). I show some solutions:

{1 = 0., 2 = 0., 3 = 0., 4 = 2.21}, {1 = 0., 2= 1.57, 3= 1.57, 4= -2.50},

{1= 3.14., 2= 2.65, 3 = 0., 4= -2.21}, {1= 0., 2= 3.14, 3= 2.83, 4= 2.06},

{1= 3.14, 2= 3.14, 3= 0.614, 4= 2.05}, {1= 0., 2= -1.57, 3= 1.71, 4= = 1.84}

To make sure all solutions and code are right. I try calculating again with MATLAB:

clear all

alpha1 = 0., theta2 = 0., theta3 = 0., theta4 = 2.21;

a1=1; a2=1;a3=3;a4=2;d6=2;

x=a1+a3*cos(theta2+theta3)+a2*cos(theta2)+a3*cos(theta2+theta3+theta4)+d6*cos(theta2+theta3+theta4)

y=cos(alpha1)*(a3*sin(theta2+theta3)+a2*sin(theta2)+a3*sin(theta2+theta3+theta4)+d6*sin(theta2+theta3+theta4))

z=sin(alpha1)*(a3*sin(theta2+theta3)+a2*sin(theta2)+a3*sin(theta2+theta3+theta4)+d6*sin(theta2+theta3+theta4))

As we can see that the results are good.

## 4.2 THE SOLUTION FOR θ6

ω = θ6 + α1

θ6= ω - α1

ω – desired orientation of wheel in z-axis

From results we should remove ones with imaginary parts. As we can see the arm sometimes is able to obtain more than one result.

# 5.Simple control

## 5.1 ROS and V-REP

In laboratory classes I have also made a simple control in ROS and V-REP program.

I control the arm via Sony PS5 Pad Controller.

1st joint control axis

2nd joint control axis

3rd joint control axis

4th joint control axis

5th joint control axis

To control the simulation in V-REP I had to create a node thanks which it would be possible. The code of the node is included in Appendix 2.

## 5.2 Arduino with Servos

To control Robot. I use Arduino UNO R3, Module Bluetooth HC-05 with Servos.

Connect Module Bluetooth with phone via app Bluetooth RC Controller.

Scheme

With this base we can connect to many servos to control. Due to the limited port of the Arduino, the Arduino can control up to 6 servos at the same time.

This is very convenient because it is remote control via Bluetooth

The code for Arduino in Appendix 3.

# 6.Model

# 7.APPENDIX

## 7.1 MATLAB CODE

clear all

close all

clc

syms a1 a2 a3 a4 ;

syms d6;

syms alpha1 theta2 theta3 theta4 theta6;

%system 1

Tranx_a1=[1 0 0 a1;0 1 0 0;0 0 1 0;0 0 0 1];

Rotx_alpha1=[1 0 0 0;0 cos(alpha1) -sin(alpha1) 0; 0 sin(alpha1) cos(alpha1) 0;0 0 0 1];

%system 2

Rotz_theta2=[cos(theta2) -sin(theta2) 0 0;sin(theta2) cos(theta2) 0 0;0 0 1 0; 0 0 0 1];

Tranx_a2=[1 0 0 a2;0 1 0 0;0 0 1 0;0 0 0 1];

%system 3

Rotz_theta3=[cos(theta3) -sin(theta3) 0 0;sin(theta3) cos(theta3) 0 0;0 0 1 0; 0 0 0 1];

Tranx_a3=[1 0 0 a3;0 1 0 0;0 0 1 0;0 0 0 1];

%system 4

Rotz_theta4=[cos(theta4) -sin(theta4) 0 0;sin(theta4) cos(theta4) 0 0;0 0 1 0; 0 0 0 1];

Tranx_a4=[1 0 0 a3;0 1 0 0;0 0 1 0;0 0 0 1];

%system 5

Rotz_90=[0 -1 0 0;1 0 0 0;0 0 1 0;0 0 0 1];

Rotx_90=[1 0 0 0;0 0 -1 0;0 1 0 0;0 0 0 1];

%system 6

Rotz_theta6=[cos(theta6) -sin(theta6) 0 0;sin(theta6) cos(theta6) 0 0;0 0 1 0; 0 0 0 1];

Tranz_d6=[1 0 0 0;0 1 0 0;0 0 1 d6;0 0 0 1];

%Danavit-Hartenberg Notations

A1=Tranx_a1*Rotx_alpha1;    A1=simplify(A1)

A2=Rotz_theta2*Tranx_a2;    A2=simplify(A2)

A3=Rotz_theta3*Tranx_a3;    A3=simplify(A3)

A4=Rotz_theta4*Tranx_a4;    A4=simplify(A4)

A5=Rotz_90*Rotx_90         %A5=simplify(A5)

A6=Rotz_theta6*Tranz_d6;    A6=simplify(A6)

%Forward kinematic for each joint

T01=A1;

T02=T01*A2;

T03=T02*A3;

T04=T03*A4;

T05=T04*A5;

T06=T05*A6;

T06=simplify(T06)

## 7.2 ROSNODE CODE

#include <ros/ros.h>

#include <std_msgs/Float64.h>

#include <sensor_msgs/Joy.h>

ros::Publisher servo1_pub;

ros::Publisher servo2_pub;

ros::Publisher servo3_pub;

ros::Publisher servo4_pub;

ros::Publisher servo5_pub;

void joyCallback(const sensor_msgs::Joy::ConstPtr & joy) {

double first_joint =joy->axes[0]

double second_joint = joy->axes[1];

double third_joint = joy->axes[2];

double fourth_joint = joy->axes[3];

double fifth_joint = joy->axes[4];

std_msgs::Float64 servo1;

std_msgs::Float64 servo2;

std_msgs::Float64 servo3;

std_msgs::Float64 servo4;

std_msgs::Float64 servo5;

servo1.data=first_joint;

servo2.data=second_joint;

servo3.data=third_joint;

servo4.data=fourth_joint;

servo5.data=fifth_joint;

servo1_pub.publish(servo1);

servo2_pub.publish(servo2);

servo3_pub.publish(servo3);

servo4_pub.publish(servo4);

servo5_pub.publish(servo5);

}

int main (int argc, char **argv) {

ros::init(argc,argv,"joy_controller_node");

ros::NodeHandle nh_("~");

std::string servo1_name;

std::string servo2_name;

std::string servo3_name;

std::string servo4_name;

std::string servo5_name;

std::string servo1_string;

std::string servo2_string;

std::string servo3_string;

std::string servo4_string;

std::string servo5_string;

nh_.param<std::string>("first_motor_name",servo1_name,"RobotArm_servo1");

nh_.param<std::string>("second_motor_name",servo2_name,"RobotArm_servo2");

nh_.param<std::string>("third_motor_name",servo3_name,"RobotArm_servo3");

nh_.param<std::string>("fourth_motor_name",servo4_name,"RobotArm_servo4");

nh_.param<std::string>("fifth_motor_name",servo5_name,"RobotArm_servo5");

servo1_string.append("/vrep/")

servo1_string.append(servo1_name);

servo2_string.append("/vrep/")

servo2_string.append(servo2_name);

servo3_string.append("/vrep/")

servo3_string.append(servo3_name);

servo4_string.append("/vrep/")

servo4_string.append(servo4_name);

servo5_string.append("/vrep/")

servo5_string.append(servo5_name);

ros::Subscriber joy_sub;

joy_sub = nh_.subscribe<sensor_msgs::Joy>("joy",1,joyCallback);

servo1_pub = nh_.advertise<std_msgs::Float64>(servo1_string,1);

servo2_pub = nh_.advertise<std_msgs::Float64>(servo2_string,1);

servo3_pub = nh_.advertise<std_msgs::Float64>(servo3_string,1);

servo4_pub = nh_.advertise<std_msgs::Float64>(servo4_string,1);

servo5_pub = nh_.advertise<std_msgs::Float64>(servo5_string,1);

ros::Rate loop_rate(5);

while (ros::ok()){

ros::spinOnce();

loop_rate.sleep();

}

}

## 7.3 ARDUINO CODE

#include <SoftwareSerial.h>
#include <Servo.h>
SoftwareSerial HC05(2,3);
char tem;
Servo myservo1;
int servoPin1=9;
int angle1=0;
Servo myservo2;
int servoPin2=10;
int angle2=0;
void setup (){
  Serial.begin (115200) ;
  HC05.begin (9600) ;
  myservo1.attach(servoPin1);
  myservo2.attach(servoPin2);
}
void loop() {
  if (HC05.available ()) {
    tem= HC05.read () ;
  }
switch (tem) {
  case 'F':
  {
  angle1=angle1+1;
  break;
  }
  case 'B':
  {
  angle1=angle1-1;
  break;
  }
  case 'L':
  {
  angle2=angle2+1;
  break;
  }
  case 'R':
  {
  angle2=angle2-1;
  break;
  }
  }
   Serial.println (tem) ;
   myservo1.write(angle1);
   delay(5);
   myservo2.write(angle2);
   delay(5);
}